Implementing VPN Solutions with FortiGate

Executive Summary

This project aimed to design and deploy a secure, resilient network infrastructure connecting a Headquarters (HQ) and a remote Branch office. Using Fortinet technologies (FortiGate VMs), the project successfully implemented remote access solutions (SSL VPN), site-to-site connectivity (IPsec), network optimization (SD-WAN), and unified threat management (UTM). The final topology ensures high availability with automated failover and strict security policies.

1. Project Topology & Design

- Infrastructure: Virtualized environment using VMware Workstation.

- Nodes: Two FortiGate Firewalls (HQ & Branch), two Ubuntu Linux endpoints.

- Network Segments:

  - HQ LAN: 10.0.11.0/24

  - Branch LAN: 10.0.22.0/24

  - WAN 1 (ISP 1): Simulated Internet (NAT Mode).

  - WAN 2 (ISP 2): Simulated Private Link (LAN Segment - 172.16.1.1).

## Remote Access Configuration (SSL VPN)

To support remote workers, an SSL VPN solution was deployed at the HQ.

- Configuration: Tunnel Mode with Split Tunneling enabled to optimize bandwidth.

- Authentication: Local user database with specific user groups (SSL_VPN_USERS).

- Testing: Successful connection established via FortiClient from an external host. Access verified via Ping to internal HQ resources.

## Site-to-Site Connectivity (IPsec VPN)

A secure tunnel was established to connect the Branch office to the HQ.

- Protocol: IKEv2 with Pre-Shared Key authentication.

- Redundancy: Two distinct tunnels were created:

  - To-HQ-1 (via Public Internet).

  - To-HQ-2 (via Private Link).

- Status: Both tunnels achieved "UP" status in the IPsec Monitor.

## SD-WAN Integration & Optimization

To manage the dual tunnels efficiently, SD-WAN was implemented.

- Zone Configuration: Both IPsec tunnels were mapped to a single VPN_Zone.

- Performance SLA: A health-check monitor (Check-HQ) was configured using Ping to measure Latency and Packet Loss in real-time.

- Rules: Traffic steering rules were configured to select the "Best Quality" link.

- Failover Test: The primary WAN link was manually disabled. Traffic successfully failed over to the secondary link without interrupting active sessions.

## Security & Threat Management

To secure the open tunnels, security profiles were applied to firewall policies.

- Web Filtering: A Static URL Filter was applied to block access to specific unauthorized domains (e.g., social media).

- Verification: Attempts to access blocked sites from the Branch client resulted in a Fortinet "Web Page Blocked" warning.

## Conclusion

The project successfully met all objectives. The resulting network is secure, redundant, and optimized for performance. Future recommendations include integrating LDAP/Radius for centralized authentication and deploying FortiAnalyzer for advanced logging